/*			ellf.c

*

* This program calculates design coefficients for

* elliptic function digital filters.

*

* Inputs are entered by keyboard, as follows:

*

* Type of filter (1: low pass, 2: band pass, 3: high pass,

*                 4: band reject, 0: exit to monitor)

* Order of filter (an integer)

* Passband ripple (peak to peak decibels)

* Sampling frequency (Hz)

* Passband edge frequency (Hz)

* Second passband edge frequency (for band pass or reject filters)

* Stop band edge frequency (Hz)

*      or stop band attenuation (entered as -decibels)

*

* The "exit to monitor" type 0 may be used to terminate the

* program when input is redirected to come from a command file.

*

* If your specification is illegal, e.g. the stop band edge

* is in the middle of the passband, the program will make you

* start over.  However, it remembers and displays the last

* value of each parameter entered.  To use the same value, just

* hit return instead of typing it in again.

*

* The program displays all pass band and stop band edge frequencies,

* and the stop band attenuation. The z-plane coefficients are

* printed in these forms:

*   Numerator and denominator z polynomial coefficients

*   Pole and zero locations

*   Polynomial coefficients of quadratic factors

*

* The following mathematical subroutines are required:

* asin(), atan(), atan2(), cos(), ellik()

* ellpj(), ellpk(), exp(), log(), sin(), sqrt().

* Of these, all but the elliptic functions are available in

* standard C function libraries.

*

* References:

* A. H. Gray, Jr., and J. D. Markel, "A Computer Program for

* Designing Digital Elliptic Filters", IEEE Transactions on

* Acoustics, Speech, and Signal Processing 6, 529-538

* (December, 1976)

*

* B. Gold and C. M. Rader, Digital Processing of Signals,

* McGraw-Hill, Inc. 1969, pp 61-90

*

* M. Abramowitz and I. A. Stegun, eds., Handbook of Mathematical

* Functions, National Bureau of Standards AMS 55, 1964,

* Chapters 16 and 17

*

*

*		ellf.c

*

*    System dependent aspects:

*

* This program has been tested on IBM PC with Microsoft C;

* DEC PDP-11 with Whitesmiths C; and DEC VAX with VAX C.

* All answers agreed within 1 in the last reported decimal place.

*

* It was also tried on a Unisoft 68000 Unix system, but did

* not work.  The reason was traced to a serious problem

* in Unisoft's floating point arithmetic -- so that

* particular attempt was abandoned.

*

* The code was compiled without any complaints by the

* Greenhills 68000 cross compiler running on a VAX;

* but the object code was not tested.

*

* You need: this file; the three elliptic functions ellik.c,

* ellpk.c, and ellpj.c; and perhaps const.c (possibly named

* mthutl.c).  If you don't have the necessary constants,

* define UNK below to be 1 and hope your C compiler converts

* the decimal values correctly.

*

* See the accompanying output listing file ellf.ans

* for a set of correct answers.  If the low pass and high

* pass options work but the others don't, then examine your

* atan2() function carefully for reversed arguments or perhaps

* an offest of pi.

*

* On most systems, define DECPDP to be 0.  This should work

* with most Unix-like function libraries.

*

* All static variables are initialized in order to make

* the Whitesmiths C compiler happy.

*

* The program has been cut up into rather arbitrary subroutines

* to get it safely through the Microsoft C compiler on IBM PC.

*

* Function abs(x) returns the double precision absolute

* value of x.  Function polevl() is needed by ellpk();

* Both abs() and polevl() are included with this listing.

*

*

* Steve Moshier, December 1986

*/