ZX-ATKEY - V.2  (c) 2005  wilf rigter

A ZX81 interface for the AT keyboard

After an 8 year hiatus, I can finally finish the AT keyboard to ZX81 interface.

The PC printer port of the original ROMKEY article has now been replaced

with a PICAXE 18X chip that is programmable in BASIC and has build in

support for the AT keyboard. This is also my first PICAXE project and I am

happy to report that the PICAXE hardware and documentation is easy to use

and the software is easy to program.  The BASIC KEYIN and KEYLED functions

make interfacing the keyboard and decoding the messy protocol reasonably

straight forward.

The 18X ZX-ATKEY design allows use of an AT keyboard as a transparent

substitute for the old ZX81 membrane keyboard. In addition, the PICAXE

has a true asynchronous port that allows the ZX81 to communicate with

a PC at up to 4800 baud. The hardware for this project is amazingly simple

consisting of a PICAXE 18X and a 27C256 EPROM and some passive components.

I build the ZX-ATKEY circuit on a prototype board available from Peter Anderson,

which I modified slightly to mount the PS2 connector next to the DB9 and DC

power supply connectors.

THE ZX-ATKEY CIRCUIT

5V                    5V  5V

ZX81 PCB REAR        ZX/ASCII ROM    |                     |   |     AT KBD

EDGE CONNECTOR         27C256       1M        PICAXE18X  4.7K 4.7K    PLUG

______________    | 0.1    _________    |   |

+5V  ---> 5V      |           CE |20-+-||--13|OUT7     |   |   |      DATA

D0   -----------11| D0        A8 |25--------6|OUT0  IN7|16-+---|--------o

D1   -----------12| D1        A9 |24--------7|OUT1  IN6|15_    |   0V-o   o-5V

D2   -----------13| D2       A10 |21--------8|OUT2     |   `---+------o   o

D3   -----------15| D3       A11 |23--------9|OUT3     |            CLOCK

D4   -----------16| D4       A12 |2--------10|OUT4     |

A8   -----------10| A0       A13 |26-------11|OUT5     |

A9   ------------9| A1       A14 |27-------12|OUT6     |

A10  ------------8| A2           |           |         |

A11  ------------7| A3        D5 |17-------17|IN0      |

A12  ------------6| A4        D6 |18-------18|IN1      |

A13  ------------5| A5        D7 |19--------3|IN2      |

A14  ------------4| A6           |           |     SOUT|2--180---- SERIAL OUT

A15  ------------3| A7           |    5V     |      SIN|3--22K--+- SERIAL IN

A0   -->|--+----22|OE         Vdd|28---|---14|Vdd      |        |

1N4448 |      |           Vpp|1____|     |    RESET|4 __   10K

RD   -->|--+      |_____Vss______|     |     |___Vss___|    |   |

|             |14           |          |5        |  0V

IORQ -->|--+---[4.7K]----|            === 0.1     |         |

0V   ____________________|_____________|__________|         |

RST  _______________________________________________________|

HOW ZX-ATKEY WORKS

The ZX81 uses the Z80 address lines A8-A15 to scan the membrane

keyboard matrix by pulling each keyboard row low with an address line

and reading the column data through the ULA keyboard port on  D0-D4.

The I/O address for the keyboard port is FEh.

The AT keyboard transmits serial data together with a clock signal to

the PICAXE IN6 and IN7 lines.  This data is sent in the form of a unique

8 bit SCANCODE value together with START, STOP and PARITY bits. The

PICAXE receives the serial data, checks for transmission errors and

translates the AT keyboard SCANCODE into a 7 bit ASCII value which is

output on OUT0-OUT5 together with an active low "keydown" signal on

OUT7.

The ZX/ASCII EPROM converts the ASCII code, which is connected to

EPROM address lines A9-A12 to address a block of 8 bytes containing

the ZX81 scan code. The OUT7 pin is connected via a R/C network to

pulse CE active low for 100ms when a key is pressed after which CE

returns to inactive High. This R/C conditioned pulse is necessary so that

the PICAXE program can quickly return to the KEYIN routine which waits

for the next key code transmitted by the AT keyboard.

The ZX81 addresses the EPROM when the normal ZX keyboard I/O port

FEh is read with an active low on the EPROM OE line and by scanning the

8 EPROM bytes with the A8-A15 lines connected to the EPROM A0-A7 pins.

With CE pin low and OE pin low the EPROM D0-D4 data lines are enabled

and read on the ZX81 data bus in parallel with the ULA keyboard data.

This ZX-ATKEY project is a work in progress and will be completed in

stages. Here are some pictures that show the PICAXE protoboard with

the PS2 connector added.

I am currently experimenting with a short program by Stan Swan

called keyb18xa.bas that demonstrates the keyboard function and

ASCII conversion to test the keyboard interface on the protoboard.

Next the EPROM hardware and ZX81 interface which was

previously tested will be added similar to the 16C84 version.

Then I will modify the PICAXE code to access the EPROM instead of

the serial port at which point the interface can be tested with a ZX81.

Then the code for ZX81 control of the PIC serial port and the code for

data communication using the EPROM D5-D7 lines will be added.

I will try using a blog style progress report. But each stage will be

documented  posted as a series of updates to this article on the

ZX-ATKEY project.

enjoy

wilf